# PoE Switch

## 🔌 What is a PoE Switch?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

A PoE Switch (Power over Ethernet Switch) is a network switch that not only transfers data, but also delivers electrical power to connected devices through the same Ethernet cable.

It allows you to power devices like:

- IP cameras

- VoIP phones

- Wireless Access Points (WAPs)

- IoT sensors

without needing separate power adapters or outlets.

## ⚙️ How PoE Works

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

Normally, an Ethernet cable carries only data. With PoE, the switch injects DC power into the same cable, allowing devices to receive both data and power through a single line.

[PoE Switch]

├───┬───┬──────┬─────

↓   ↓   ↓      ↓

[IP Cam] [Phone] [WAP] [Printer]

(PoE)   (PoE)   (PoE) (No PoE)

- Devices like printers that don’t support PoE just receive data — safely.

## 🔧 Key Features of a PoE Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Feature | Description |
| --- | --- |
| Power + Data | Supplies electricity and network access over one cable. |
| Simplified Setup | No need for power outlets near each device. |
| Cost Saving | Reduces wiring and installation complexity. |
| Safe Power Delivery | Auto-detects PoE-compatible devices and prevents damage to non-PoE devices. |
| Supports PoE Standards | Follows IEEE standards like 802.3af (15.4W), 802.3at (30W), or 802.3bt (up to 60W/100W). |

## 🆚 PoE vs Regular Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Feature | PoE Switch | Standard Switch |
| --- | --- | --- |
| Power Delivery | ✅ Yes | ❌ No |
| Devices Powered | IP Cams, Phones, WAPs | Must use separate power |
| Setup | Easier (fewer cables) | Requires both data + power |
| Cost | Slightly higher | Lower |

## 🧱 Types of PoE Switches

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Type | Description |
| --- | --- |
| Unmanaged PoE Switch | Plug-and-play, no configuration needed. |
| Managed PoE Switch | Includes VLAN, traffic control, remote monitoring, and port power settings. |
| Passive PoE Switch | Delivers fixed voltage regardless of device — use with caution. |
| Active PoE Switch | Negotiates power safely using PoE standards — recommended. |

## 🔌 Power Budget Example

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

If your switch supports 120W PoE power, you can power:

- 4 x IP Cameras @ 15W = 60W

- 2 x Access Points @ 25W = 50W

- Total = 110W used (within budget)

Always check the PoE budget to ensure you don’t exceed it.