# Layer 3 Switch

## 🌐 What is a Layer 3 Switch?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

A Layer 3 Switch is a network switch that combines the speed of a Layer 2 switch with the routing capabilities of a router. It operates at both:

- Layer 2 (Data Link Layer) — for switching using MAC addresses

- Layer 3 (Network Layer) — for routing using IP addresses

So, a Layer 3 switch can switch traffic within VLANs and also route traffic between VLANs or subnets, without needing a separate router.

## 🧠 Why It Matters

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

In large or segmented networks, you often need to:

- Keep traffic isolated (e.g. between departments)

- But still allow controlled communication (e.g. HR talking to IT)

A Layer 3 switch makes that possible, without bottlenecks from a traditional router.

## 🔧 Key Features of a Layer 3 Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Feature | Description |
| --- | --- |
| Inter-VLAN Routing | Routes traffic between VLANs internally. |
| IP-Based Forwarding | Uses IP addresses and routing tables (like a router). |
| High-Speed Switching | Faster than traditional routers (done in hardware). |
| Supports Routing Protocols | Some support static routes, RIP, OSPF, BGP, etc. |
| Access Control | Can apply ACLs (Access Control Lists) for security. |
| Supports VLANs | Create, manage, and route between multiple VLANs. |

## 🖥️ Example Network Setup

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

[Layer 3 Switch]

┌───────────────┐

VLAN 10       VLAN 20       VLAN 30

(HR Dept)     (IT Dept)     (Guests)

[PCs]         [Servers]     [Wi-Fi]

- All VLANs are isolated

- Layer 3 switch routes between them, with rules controlling what’s allowed

## 🆚 Layer 2 vs Layer 3 Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Feature | Layer 2 Switch | Layer 3 Switch |
| --- | --- | --- |
| Works At | Data Link Layer (Layer 2) | Network Layer (Layer 3) |
| Forwards By | MAC Address | IP Address |
| Inter-VLAN Routing | ❌ Needs router | ✅ Built-in |
| VLAN Support | ✅ Yes | ✅ Yes |
| Static Routing | ❌ No | ✅ Yes |
| Routing Protocols | ❌ No | ✅ Sometimes (depends on model) |
| Cost | 💲 Lower | 💲💲 Higher |

## ✅ Pros of Layer 3 Switches

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

- ✔ High-speed inter-VLAN routing

- ✔ Reduces need for separate routers

- ✔ Improves network performance

- ✔ Great for campus, enterprise, and data center networks

- ✔ Often includes redundancy, QoS, and security controls

## ❌ Cons

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

- ❗ More expensive than basic Layer 2 switches

- ❗ Not all Layer 3 switches support advanced routing protocols

- ❗ Can be overkill for small networks

## 📦 Common Brands & Examples

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Brand | Layer 3 Switch Models |
| --- | --- |
| Cisco | Catalyst 9300, 9500 |
| Juniper | EX4300, QFX Series |
| HP/Aruba | 2930F, 5400R |
| Netgear | ProSAFE L3 models |
| MikroTik | CRS series (some L3 features) |