# Power over Ethernet (PoE)

## ⚡ What Is Power over Ethernet (PoE)?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

Power over Ethernet (PoE) is a technology that allows electrical power and data to be delivered over a single Ethernet cable (typically Cat5e or higher). This means you can power devices like cameras, access points, or VoIP phones without needing a separate power supply.

## Power over Ethernet (PoE) Image

# Source: ScreenShot Source: Provided by: [Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> Tool: [Lightshot Program] -> From [Site: Google Search: Images: Power over Ethernet (PoE)] on [12/18/2025]

## 📦 Belongs to:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

✅ Types of Network Cables / Network Features

## 🧠 Key Concept

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

PoE combines data transmission and DC power delivery on standard twisted-pair Ethernet cables.

## 🔌 How It Works

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Component | Role |
| --- | --- |
| PSE (Power Sourcing Equipment) | Supplies power (e.g. PoE switch or injector) |
| PD (Powered Device) | Receives power (e.g. IP camera, Wi-Fi AP) |
| Cable Type | Cat5e, Cat6, Cat6a (typically) |
| Connector | RJ-45 |

## 🔋 PoE Standards

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Standard | Power per Port | Max Cable Length | Used For |
| --- | --- | --- | --- |
| IEEE 802.3af (PoE) | Up to 15.4 W | 100 meters | IP phones, simple APs |
| IEEE 802.3at (PoE+) | Up to 30 W | 100 meters | HD cameras, dual-band APs |
| IEEE 802.3bt (PoE++) | Up to 60 W (Type 3) / 100 W (Type 4) | 100 meters | PTZ cameras, PoE lighting, thin clients |

## 🧾 Common PoE Devices

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

- 📷 IP Cameras (especially in security systems)

- 📶 Wireless Access Points (WAPs)

- ☎️ VoIP Phones

- 🚪 Door access control systems

- 💡 PoE-powered LED lights

## ✅ Advantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

- 🔌 Single cable for power + data = less wiring

- 🧰 Easy installation (especially in hard-to-reach places)

- 🔄 Centralized power management via PoE switches

- ⚡ Safe and intelligent — only delivers power when a device is PoE-compatible

## ❌ Disadvantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

- 📏 Still limited to 100m cable length

- ⚙️ Higher power devices (e.g., computers) need more than PoE offers

- 💰 PoE switches and injectors can be more expensive

## 📌 Summary

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Attribute | Value |
| --- | --- |
| Technology | Combines data + power in Ethernet cable |
| Standards | IEEE 802.3af / 802.3at / 802.3bt |
| Cables Used | Cat5e, Cat6, Cat6a |
| Connector | RJ-45 |
| Typical Use | APs, IP cameras, VoIP, access control |