# Management Wi-Fi frames

## 🧱 What Is Management Wi-Fi frames?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Management Wi-Fi frames are one of the three main categories of IEEE 802.11 (Wi-Fi) frames. They are used to establish, maintain, and terminate wireless connections, rather than to carry user data.

## Wi-Fi Frame Types (Context)

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Wi-Fi frames fall into three groups:

- Management frames – control the connection lifecycle

- Control frames – help deliver data reliably

- Data frames – carry actual user data

## What Management Frames Do

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Management frames handle tasks such as:

- Discovering Wi-Fi networks

- Authenticating devices

- Associating and disassociating clients

- Maintaining connection parameters

They are essential for a device to join and stay connected to a Wi-Fi network.

Common Management Wi-Fi Frames

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

1. Beacon

- Sent periodically by Access Points (APs)

- Advertises the network (SSID, security type, supported rates)

- Lets devices discover available Wi-Fi networks

2. Probe Request

- Sent by a client searching for networks

- Can be broadcast or targeted to a specific SSID

3. Probe Response

- Sent by an AP in reply to a probe request

- Provides network capabilities and details

4. Authentication

- Verifies a device’s identity before joining

- Can be open (no real authentication) or secure

5. Association Request

- Sent by a client to join an AP

- Includes device capabilities

6. Association Response

- AP’s reply accepting or rejecting the client

7. Disassociation

- Terminates an association but allows re-association

8. Deauthentication

- Completely ends authentication and association

## Security Note

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Traditionally, management frames were unencrypted, making them vulnerable to attacks (e.g., deauthentication attacks).
To address this, 802.11w (Protected Management Frames, PMF) encrypts certain management frames to improve security.

## In Simple Terms

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Management Wi-Fi frames are the “handshake and housekeeping messages” that allow Wi-Fi devices to find networks, connect, stay connected, and disconnect properly.