# Control Wi-Fi frames

## 🧱 What Is Control Wi-Fi frames?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Control Wi-Fi frames are one of the three main IEEE 802.11 frame types. Their purpose is to manage access to the wireless medium and ensure reliable delivery of data frames.

They do not carry user data and do not establish connections (that’s the job of management frames). Instead, they coordinate how and when data is sent.

## Wi-Fi Frame Types (Quick Context)

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

- Management frames – discover and join networks

- Control frames – coordinate transmission and reliability

- Data frames – carry actual user traffic

## What Control Frames Do

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Control frames help with:

- Avoiding collisions on the wireless channel

- Acknowledging successful frame delivery

- Managing who can transmit and when

- Supporting power-saving mechanisms

## Common Control Wi-Fi Frames

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

1. RTS (Request to Send)

- Sent by a device before transmitting large data frames

- Asks for permission to use the channel

- Helps prevent collisions (hidden node problem)

2. CTS (Clear to Send)

- Sent in response to RTS

- Grants permission to transmit

- Other devices hearing CTS stay silent

3. ACK (Acknowledgment)

- Confirms successful receipt of a frame

- If not received, the sender retransmits

4. Block ACK

- Acknowledges multiple frames at once

- Improves efficiency for high-throughput traffic

5. PS-Poll (Power Save Poll)

- Used by devices in power-saving mode

- Requests buffered data from the Access Point

## Example Flow (Simple)

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

- Device sends RTS

- AP replies with CTS

- Device sends Data frame

- AP replies with ACK

This sequence ensures orderly and reliable communication on a shared wireless medium.

## Why Control Frames Matter

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

- Wi-Fi is half-duplex (only one device transmits at a time)

- Control frames prevent collisions and wasted airtime

- They significantly improve network performance and reliability

## In Simple Terms

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Control Wi-Fi frames are the “traffic signals” of Wi-Fi, making sure devices take turns and confirming that messages arrived safely.