# Static Route

## 🧱 What Is Static Route?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

A static route is a manually configured path in a device's routing table. Unlike dynamic routes, it does not change automatically and must be added, modified, or removed by a network administrator.

## 🧠 Simple Definition:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

A static route tells the router:
 “To reach this destination network, send packets via this specific next-hop IP or interface — and don’t learn it from routing protocols.”

## 🧾 Example:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

You want to route traffic to network 10.10.20.0/24 through a next-hop router at 192.168.1.2.

# Linux/Unix CLI:

ip route add 10.10.20.0/24 via 192.168.1.2 dev eth0

## Routing Table Entry:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Destination | Netmask | Next Hop | Interface |
| --- | --- | --- | --- |
| 10.10.20.0 | 255.255.255.0 | 192.168.1.2 | eth0 |

## 🔍 Characteristics of Static Routes:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Feature | Description |
| --- | --- |
| ⚙️ Manually configured | Set by a network admin |
| 🛑 Does not adapt | Does not change if the network changes |
| ⚡ Fast & efficient | Low CPU/memory use (no routing protocol overhead) |
| 🔄 Commonly used for | Default routes, backup routes, or small networks |
| 🔥 Fails silently | If next hop is unreachable, traffic may be dropped |

## 🖥 Example on Cisco Router:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Router(config)# ip route 10.0.0.0 255.255.255.0 192.168.1.1

This tells the router: "To reach 10.0.0.0/24, send packets to 192.168.1.1."

## ✅ Summary

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Property | Static Route |
| --- | --- |
| Configured by | Admin (manual) |
| Next Hop | Required |
| Adapts to Changes | ❌ No |
| Best Use Case | Small networks, default route, failover paths |