# ARP Entries – Static

## 🧱 What Is ARP Entries – Static?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

A static ARP entry is a manually configured mapping between an IP address and a MAC address. It is fixed and does not change or expire, even if the device is rebooted or the network changes.

## 🧠 Simple Definition:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

A static ARP entry is a permanent IP-to-MAC mapping manually added to a device's ARP table by an administrator.

## 📋 Example: Static ARP Entry

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| IP Address | MAC Address | Interface | Type |
| --- | --- | --- | --- |
| 192.168.1.20 | AA:BB:CC:11:22:33 | eth0 | Static |

## 🔧 How to Add Static ARP Entries

### ▶ On Linux:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

sudo ip neigh add 192.168.1.20 lladdr aa:bb:cc:11:22:33 dev eth0 nud permanent

### ▶ On Windows:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

arp -s 192.168.1.20 aa-bb-cc-11-22-33

### ▶ On Cisco IOS:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

arp 192.168.1.20 aa-bb-cc-11-22-33 ARPA

## 🛡️ Use Cases for Static ARP

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Scenario | Why Use Static ARP? |
| --- | --- |
| Prevent ARP spoofing (security) | Hard-coded mapping can't be poisoned |
| Mission-critical device (e.g., gateway) | Ensure reliable communication |
| Devices without ARP (printers, etc.) | Needed to communicate on a Layer 2 level |

## 🔍 Key Characteristics of Static ARP

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Feature | Static Entry |
| --- | --- |
| ⚙️ Configured by | Manually by admin |
| 🧠 Auto-learned | ❌ No |
| 🕓 Expires | ❌ Never |
| 💾 Saved on reboot | ✅ Usually (depending on OS) |
| 🔁 Updated automatically | ❌ Never |

## ✅ Summary

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Attribute | Static ARP Entry |
| --- | --- |
| How it's added | Manually |
| Lifetime | Permanent |
| Secure? | Yes – cannot be changed by ARP spoofing |
| Used for | Security, reliability, special devices |