# Hardware Redundancy

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/21/2026]

Hardware Redundancy is the first and most common type of redundancy used for fault tolerance in Real-Time Systems (RTS). It involves duplicating physical components to ensure the system continues operating even if some fail.

## Definition

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/21/2026]

Hardware redundancy is the technique of adding extra physical components or devices so that if one component fails, another can take over, maintaining system reliability and availability.

- Ensures continuous operation in safety-critical and real-time systems.

- Often used in aviation, aerospace, industrial automation, and automotive systems.

## Key Features

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/21/2026]

- Duplication or Triplication

  - Components such as processors, sensors, or power supplies are duplicated (2x) or triplicated (3x).

- Fault Detection via Comparison

  - Outputs from multiple components are compared to detect failures.

- Automatic Failover

  - When a component fails, the system automatically switches to the backup component.

## Common Techniques

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/21/2026]

- Dual Modular Redundancy (DMR)

  - Two identical components perform the same function

  - If outputs differ → system detects fault

- Triple Modular Redundancy (TMR)

  - Three identical components perform the same task

  - Majority voting determines correct output

  - Can tolerate one faulty component

- N-Modular Redundancy (NMR)

  - Extension of TMR with N components

  - Tolerates multiple faults depending on N

## Example

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/21/2026]

- Aerospace flight control system:

  - Three redundant processors compute control commands

  - Outputs are compared via a majority voting system

  - If one processor fails → the other two maintain correct operation

## Advantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/21/2026]

- Increases system reliability and availability

- Detects hardware failures quickly

- Essential for hard real-time and safety-critical systems

## Disadvantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/21/2026]

- High cost (duplicating/triplicating components)

- Increases weight and power consumption (important in embedded/aerospace systems)

- Adds complexity in monitoring and synchronization

## Applications

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/21/2026]

- Aerospace and avionics (flight control systems, satellites)

- Automotive systems (brake, steering, airbag control)

- Industrial automation (robotics, assembly lines)

- Medical devices (life-support systems)